# Surface Grinder Setup & Operation Checklist

## Step 1: Job Planning

- [ ] Review print or drawing thoroughly.

- [ ] Identify critical surfaces and tolerances.

- [ ] Determine material type and dimensions.

- [ ] Select appropriate grinding wheel (material, grit, hardness).

- [ ] Plan grinding sequence (rough roughing passes, then finishing passes).

## Step 2: Machine Preparation

- [ ] Inspect grinder for cleanliness and function.

- [ ] Verify coolant system (if used) is operational.

- [ ] Dress grinding wheel if necessary.

- [ ] Confirm machine guards and emergency stops are functional.

## Step 3: Workpiece Preparation

- [ ] Clean workpiece surfaces.

- [ ] Verify dimensions of raw stock.

- [ ] Deburr sharp edges if necessary.

## Step 4: Workholding Setup

- [ ] Clean magnetic chuck thoroughly.

- [ ] Place workpiece on chuck; ensure full contact.

- [ ] Activate magnetic chuck and verify secure hold.

- [ ] If necessary, use parallels or stop blocks for alignment.

## Step 5: Indicating and Zeroing

- [ ] Gently bring wheel close to part surface without contact.

- [ ] Touch off lightly to establish Z-zero.

- [ ] Plan depth of cut (light passes for finish).

- [ ] Set table travel limits for safe grinding motion.

## Step 6: Grinding Operations

- [ ] Start coolant flow (if available).

- [ ] Begin with a light spark-out pass to verify setup.

- [ ] Perform rough passes with slightly deeper cuts.

- [ ] Finish grind with lighter passes to achieve surface finish.

- [ ] Adjust crossfeed increment as needed for even coverage.

## Step 7: Measurement and Inspection

- [ ] Measure thickness and parallelism using micrometers and surface plate.

- [ ] Inspect surface finish visually and by touch.

- [ ] Compare measurements to blueprint tolerances.

- [ ] Record all findings for review.

## Step 8: Cleanup and Review

- [ ] Turn off magnetic chuck and remove part safely.

- [ ] Clean the grinder and remove all grinding dust.

- [ ] Dress wheel again if needed for next use.

- [ ] Reflect on process: What went well? What could be improved?