# Manual Mill Setup & Operation Checklist

## Objective:

To provide a structured checklist and guide for students to follow when preparing for and executing a manual milling operation, ensuring safety, accuracy, and success.

## Step 1: Job Planning

- [ ] Review print or drawing thoroughly.

- [ ] Identify critical features (faces, holes, slots).

- [ ] Determine material type and dimensions.

- [ ] Choose appropriate tooling (end mills, drills, etc.).

- [ ] Plan machining sequence (facing first, then features).

## Step 2: Machine Preparation

- [ ] Inspect machine for cleanliness and function.

- [ ] Verify vise is installed properly.

- [ ] Inspect and clean vise jaws and parallels.

- [ ] Select and install correct cutting tools.

- [ ] Check tool condition (sharpness, integrity).

## Step 3: Workpiece Preparation

- [ ] Clean workpiece surfaces.

- [ ] Verify dimensions of raw stock.

- [ ] Deburr sharp edges if necessary.

## Step 4: Workholding Setup

- [ ] Place parallels in vise (if needed).

- [ ] Set workpiece firmly against parallels.

- [ ] Lightly tighten vise, tap workpiece flat with mallet.

- [ ] Final tighten vise securely.

- [ ] Verify part is parallel and flat.

## Step 5: Indicating and Zeroing

- [ ] Use dial indicator or edge finder to locate edges.

- [ ] Set X and Y zeros accurately.

- [ ] Face the top surface and set Z zero.

- [ ] Double-check all zero references before cutting.

## Step 6: Machining Operations

- [ ] Verify RPM and feed rate settings.

- [ ] Perform facing to clean top surface.

- [ ] Machine slots, pockets, or holes as per plan.

- [ ] Drill features carefully, peck drilling if deep.

- [ ] Chamfer or deburr machined edges as needed.

## Step 7: Measurement and Inspection

- [ ] Measure critical features (dimensions, squareness).

- [ ] Use micrometers, calipers, or indicators.

- [ ] Compare measurements to blueprint tolerances.

- [ ] Record all findings for review.

## Step 8: Cleanup and Review

- [ ] Remove part and clean machine.

- [ ] Remove tooling and inspect for wear.

- [ ] Return all tools and gages to their proper locations.

- [ ] Reflect on process: What went well? What could improve?

## Optional Flowchart:

Review Drawing → Plan Sequence → Prep Machine → Prep Workpiece → Setup & Indicate → Cut Features → Inspect → Clean & Reflect