# Lesson Plan

Course: CNC Machining Course

Instructor:

Lesson Title: Manual Mill Setup & Operation

Date:

Time Needed for Lesson: 4 hours

## Objectives:

- Prepare and safely set up a manual mill for basic operations.

- Mount and secure a workpiece using a milling vise and parallels.

- Set tool height using edge finders or paper method.

- Perform facing, slotting, and drilling operations.

- Measure and inspect machined features for accuracy.

- Document setup parameters and cutting data.

## Materials Needed:

- Manual Knee Mill (Bridgeport or similar)

- Sample Block Workpieces

- End Mills (various sizes)

- Center Drills and Twist Drills

- Milling Vise with Parallels

- Edge Finder or Center Finder

- Micrometers and Calipers

- Safety Glasses and Face Shields

- Handouts / Whiteboard / TV

## Hook/Lesson Opener:

Ask students:

- Why is setup just as important as the machining itself?

- What could happen if you don't properly secure your part?

Demonstrate examples of good vs poor setup.

## Presentation:

- Safety inspection and cleaning of machine.

- Install milling vise and indicate flat.

- Mount sample block workpiece using parallels.

- Find part edges using edge finder.

- Zero X, Y, and Z axes correctly.

- Perform a facing operation across the top.

- Perform a simple slot across the workpiece.

- Spot and drill holes using layout lines or DRO.

## Video Presentations:

- Manual Milling Setup and Techniques – Suburban Tool: https://www.youtube.com/watch?v=1h2H3VNhWPU

- How to Square and Indicate a Vise on Your CNC Mill – Haas Automation Tip of the Day: https://www.youtube.com/watch?v=cPvOKX5PeBo

- How to Use an Edge Finder – Haas Automation Tip of the Day: https://www.youtube.com/watch?v=P4QkO-m3t9w&pp=0gcJCYQJAYcqIYzv

- Tramming a Mill Head – Haas Automation Tip of the Day: https://www.youtube.com/watch?v=FelgkWIXKr0&pp=0gcJCYQJAYcqIYzv

- How to Use a Dial Indicator – Haas Automation Tip of the Day: https://www.youtube.com/watch?v=Gd-UqQksN3I

## Exercise:

- Perform machine and vise setup.

- Mount and indicate workpiece.

- Set X, Y, Z zeros.

- Face the top surface of the part.

- Machine a centered slot.

- Drill two holes accurately placed.

- Measure dimensions and record results.

- Log settings: RPM, feed rate, depth of cut, tool sizes.

## Recap & Connector to Next Lesson:

- Group discussion: What made some setups better than others?

- Introduce next lesson: Basic Part Projects Using Manual Mill Techniques.

## Assignment(s):

- Watch all videos listed under "Video Presentations."

- Write down the 3 most critical lessons learned during setup today.

## Assessments:

- Instructor observation of setup and operation.

- Dimensional accuracy of finished part.

- Completion of cutting data logs and setup sheets.

- Reflection paragraph submission.

## Notes:

- Ensure proper chip evacuation during machining.

- Emphasize double-checking vise tightness and parallel use.

- Encourage slow, careful, and consistent hand movements.