# True Position Feature Control Frame Breakdown

# 📏 Dimension Callout:

• Ø0.625 ± 0.001
  - This is the size of the hole.
  - Acceptable hole diameter range: 0.624" to 0.626"

# 🧩 Feature Control Frame (FCF):

⭘
Ø0.002 | A

This is a True Position tolerance. Here's what each part means:

• ⭕ (True Position): Specifies positional tolerance
• Ø0.002: The axis of the hole must lie within a cylindrical zone of 0.002" diameter
• A: Datum A is the reference surface from which the position is controlled

# 🔍 What This Means Functionally

• You're placing a hole that is 0.625" in diameter (±0.001" tolerance).
• The location of the hole's axis must be within a 0.002" diameter cylinder, positioned relative to Datum A.
• This is regardless of the hole size — no MMC or LMC modifier is shown, so this is a RFS (Regardless of Feature Size) tolerance.

# ✅ Summary Table

| Feature | Detail |
| --- | --- |
| Hole Size | Ø0.625 ±0.001 (0.624" – 0.626") |
| Tolerance Type | True Position (⭘) |
| Positional Tolerance | Ø0.002 (cylindrical tolerance zone) |
| Modifier | None → RFS (Regardless of Feature Size) |
| Datum | A |