# Answer Key and Solutions – CNC Final Exam

# Part 1: Trigonometry & Geometry (Sample Solutions)

1. Use cos(35°) = 8 / hypotenuse → hypotenuse = 8 / cos(35°) ≈ 9.76 in

2. Use tan(20°) = 2 / x → x = 2 / tan(20°) ≈ 5.49 in

3. C = √(4² + 3²) = √(16 + 9) = √25 = 5 in

4. Chamfer length = √(.250² + .250²) = √(0.125) ≈ 0.354 in

5. θ = tan⁻¹(4/10) ≈ 21.8°

6. Use tan(12°) = X / 5 → X = 5 * tan(12°) ≈ 1.06 in

7. cos(30°) = x / 5 → x = 5 * cos(30°) ≈ 4.33 in

8. C = √(6² + 8²) = √(36 + 64) = √100 = 10 in

9. Both legs ≈ 2 in (45° right triangle, equal legs)

10. Chamfer length = √(.035² + .035²) ≈ 0.0495 in

# Part 2: Applied Machining Word Problems (Answers)

11. Moves to X2 Y3 in work offset G54

12. Returns to R-plane if G99 is active

13. 0.995 to 1.005 inches

14. Flatness

15. Yes – within 0.990 to 1.010

16. Max: .3737, Min: .3643 (UNC 3/8-16 Class 2A)

17. G83

18. Misalignment, part not in tolerance

19. Vibration and tool deflection possible

20. Applies tool length offset H01, moves Z to .1, coolant on

# Part 3: GD&T (Answers)

21. Maximum Material Condition – most material allowed

22. Form = shape alone; Orientation = angle relative to a datum

23. ⊥ (Perpendicular symbol)

24. The exact desired location of a feature

25. Surface plate + dial indicator

26. Surface = 3D area, Line = 2D cross section

27. Controls feature at multiple levels or datums

28. CMM uses probes and automation; height gauge is manual

29. Tolerance applies regardless of actual size

30. VC = MMC - position = 0.500 - 0.010 = 0.490

# Part 4: Decimal Lingo & Shop Math (Answers)

31. Five thousandths

32. Three ten-thousandths

33. 0.1

34. 0.005 to 0.015

35. 0.1 = one tenth, 0.01 = ten thousandths, 0.001 = one thousandth

36. 1.253 = 1253 thousandths

37. Yes, 0.998 is within 0.995–1.005

38. Nine ten-thousandths

39. One tenth of a thousandth = 0.0001

40. 0.050

# Part 5: Matching – Milling Machine Components (Answers)

Spindle – A
Table – B
Saddle – C
Knee – D
Quill – E

# Part 6: Matching – Vocabulary (Answers)

Angularity – A
Datum – B
Blueprint – C
Tolerance – D
Chamfer – E