# CNC Machining Final Exam – Up Academy

# Part 1: Trigonometry & Geometry (10 points)

1. A cutter moves diagonally 8 inches on the X-axis at an angle of 35°. How far does it travel along its path?

2. A drill enters a part at 20° to reach a 2-inch depth. How far horizontally must it travel?

3. What is the hypotenuse of a triangle with legs 4 in. and 3 in.?

4. Calculate the length of a 45° chamfer that’s .250 inches along X and Z.

5. A tool moves 10 inches on X and 4 inches on Y. What angle does it form?

6. In a taper of 12° over 5 inches, how far does the tool move in X?

7. Solve for the unknown side: A = 5 in., angle = 30°, find the adjacent side.

8. A triangle has sides A = 6 in., B = 8 in. Find the hypotenuse.

9. A right triangle has an angle of 45°. If one leg is 2 in., find the other leg.

10. Convert a chamfer of .035 x 45° to its true length.

# Part 2: Applied Machining Word Problems (10 points)

11. A machinist sets G54 and then uses G00 X2.0 Y3.0. Where will the cutter move?

12. If G99 is active, what Z value does the tool return to after a drilling cycle?

13. A part has a tolerance of ±.005. What are the acceptable limits of a 1.000 inch feature?

14. A surface is specified to be flat within .002. What type of GD&T control is this?

15. During inspection, the diameter measured is 1.005, and the tolerance is 1.000 ±.010. Is this acceptable?

16. You’re using a 3/8-16 UNC thread. What is the major diameter tolerance range?

17. What G-code would you use to drill three holes in a line with peck depth?

18. A work offset is set incorrectly. What defect might result?

19. The lathe part is sticking out 2.5 inches. What safety or accuracy concerns arise?

20. A G43 H01 Z.1 command is issued. What happens next?

# Part 3: Geometric Dimensioning & Tolerancing (10 points)

21. What does MMC stand for and how is it used in inspection?

22. Explain the difference between form and orientation tolerances.

23. What symbol is used to represent perpendicularity?

24. Define true position in your own words.

25. What tool would be best for measuring parallelism on a block?

26. What is the difference between profile of a surface and profile of a line?

27. What does a composite tolerance frame mean?

28. How does a CMM differ from a height gauge?

29. Explain what RFS means in tolerance analysis.

30. What is the virtual condition of a shaft with MMC of 0.500 and position tolerance of 0.010?

# Part 4: Decimal Lingo & Shop Math (10 points)

31. How would you say 0.005 in the shop?

32. Convert 0.0003 to shop lingo.

33. What is one tenth in decimal?

34. If a dimension reads .010, what is the tolerance zone between ±.005?

35. Match: 0.1 = ____, 0.01 = ____, 0.001 = ____

36. A caliper reads 1.253, how many thousandths is that?

37. A tolerance is .005, and the feature measures 0.998. Is it in spec for a 1.000 nominal?

38. 0.0009 equals how many ten-thousandths?

39. 0.0001 is how many tenths of a thousandth?

40. Convert 'fifty thousandths' to decimal form.

# Part 5: Matching – Milling Machine Components (5 points)

___ Rotating shaft that holds and drives the tool

___ Surface where workpiece is secured

___ Supports the table and allows Y-axis motion

___ Supports the saddle and moves vertically

___ Houses the spindle and moves it up/down

Choices: A. Spindle, B. Table, C. Saddle, D. Knee, E. Quill

# Part 6: Matching – Vocabulary Review (5 points)

___ Allowable deviation of an angle from a datum

___ Reference feature from which measurements are taken

___ Document showing all manufacturing instructions

___ Acceptable variation in a part's size or geometry

___ Angled cut on the edge of a part

Choices: A. Angularity, B. Datum, C. Blueprint, D. Tolerance, E. Chamfer