# Worked-Out Solutions – CNC Practice Test

1. Chamfer .100 x 45°

Solution: Length = √(.100² + .100²) = √(0.01 + 0.01) = √0.02 ≈ 0.141 in

2.  Tool travels 12 in X, 5 in Y

Solution: Angle = tan⁻¹(5/12) ≈ 22.62°

3.  Diagonal of 3x4 part

Solution: Diagonal = √(3² + 4²) = √(9 + 16) = √25 = 5 in

4.  Drill at 15°, vertical depth 1.5 in

Solution: Horizontal = 1.5 / tan(15°) ≈ 1.5 / 0.2679 ≈ 5.60 in

5. Radius of 1.250 in diameter

Solution: Radius = 1.250 / 2 = 0.625 in

6. G83: Peck 0.100, Z-1.000

Solution: 1.000 / 0.100 = 10 pecks

7. Flatness .003 in

Solution: Tools: Surface plate + dial indicator

8. G54 purpose

Solution: Sets work coordinate system origin

9. 2.500 ± .010

Solution: Range = [2.490, 2.510]

10. Decimal of 3/16

Solution: 3 ÷ 16 = 0.1875

11. 4 slots of .750

Solution: 4 x .750 = 3.000 in

12. 0.060

Solution: Shop lingo: sixty thousandths

13. 0.995 in feature vs 1.000 ±.010

Solution: Range = [0.990, 1.010]; 0.995 is within spec

14. Spindle speed 4000

Solution: Code: S4000 (used with M03)

15. Cosine 60° usage

Solution: cos(60°) = 0.5; find adjacent in right triangle

16. Move from (0,0) to (6,8)

Solution: Distance = √(6² + 8²) = √100 = 10 in

17. G90

Solution: Absolute positioning mode

18. 2.0 in stick-out

Solution: Risk: chatter, tool deflection

19. G95 F.008

Solution: Feed = 0.008 in/rev

20. 1.000 ±0.005

Solution: Range = [0.995, 1.005]

21. G00 Z2.0 after drilling

Solution: Rapid retract to Z = 2.0

22. Taper 0.5 Z over 6 in

Solution: Angle = tan⁻¹(0.5/6) ≈ 4.76°

23. Drill point at (3,4)

Solution: Radial = √(3² + 4²) = √25 = 5 in

24. G43 H02 Z.1

Solution: Apply H02 tool offset, move to Z = 0.1

25. G28

Solution: Returns axes to machine zero/home

26. Measured 0.998, tol ±0.003

Solution: Range = [0.997, 1.003]; 0.998 is in spec

27. Tangent 45°

Solution: tan(45°) = 1; used for 1:1 slope calculations

28. Bore 1.500 ± .002, part = 1.502

Solution: Range = [1.498, 1.502]; accepted

29. 30° to X, X travel = 10

Solution: Z = 10 * tan(30°) = 10 * 0.577 ≈ 5.77 in

30. Improper coolant

Solution: Overheating, tool wear, poor surface finish