Care and Feed of the Water / Oil Based Tool-X Products

After a Tool-X product is installed in the machine, you should run the machine for a three (3) to five (5) hours to fully mix the nano fluid.  (For the water-based product Tool-X recommends a mix ratio of 8:1 and refract of average 5.0 in the range of 4.6 to 5.6.)  During this initial start-up period, you may see about a ¼ inch of foaming, and the product will have a slight smell.  Don’t be concerned because these features were designed into the product and will go away in a few days!

The foam, smell, and the start-up period are to allow the two (2) different carriers (water & oil) and the nano particulates to fully mix together.  Every machine has a different capacity and an internal pump system that varies related to how active & strong it is recircling the mixture.  Never use a five (5) micron filter, we recommend using a +20-micron filter, so the nanoparticles are not removed from the mixture.

After the initial install, the recommended water-based startup procedure is to run the machine at a 50% for at least two (2) days and then start to increase speeds & feeds in 20% intervals, until you get to 80% - 90%, then increase the final 10% - 20% interval(s) - (Seven (7) days = 100%).

Another way to get a head start on getting the water-based product blended is to clean & fill it up on a Friday / Saturday and let it run until Monday morning!

During the two (2) day start-up period, you will start to see three (3) visual indicators the material is working:

- Your chips will change color & not look burnt.

- The chips will become shorter which should eliminate or reduce bird nesting.

- The surface or RA finish will improve.

We always recommend that you compare a sample of your old chips & measure the RA finish off the old coolant, so you can see these visual differences.

Tool-X recommends that you keep a few inserts (i.e., tools, etc.), collect a little bag of chips, and measure you RA prior to changing out to a Tool-X nano fluid(s) to get a great comparison after the Tool-X start-up has ended.

In the initial startup, it will take a little compassion & care, but right after those initial seven (7) days, push the machine as you see fit!  The decrease in cycle time will be in a stepping down process and may take several weeks to get to the optimal cycle time.

Over time the number of nanoparticles will increase inside the machine which in turn will continue to improve the cycle time and tool life.  True changes in tool life will occur when the dull tools are replaced with tools with new edges.

Most Tool-X customers will see, depending on the material & the refract, an improvement of at least a:

- Hundred (100) percent increase in Tool-Life.

- Twenty (20) percent in their Speeds & Feeds.

If you not getting these types of results, please call Tool-X directly or your Sales Representative, to see if we can determine a root cause.  If necessary, we can set up a conference call, meeting, or make a special visit to your facility.

After the start-up period has ended you would run Tool-X’s product in the machine just as you would have run that piece of equipment in the past and by conducting maintenance by cleaning the tank.

In a water-based application Tool-X recommends that you should keep oil out of the machine with a good skimmer.  The Carol Company brand has produced positive results working with our Nano Fluid Technology!

We hope this information helps.

Cary M. Endlich

(O) 248 303-6351