# 🔍 Blueprint Analysis & Tooling Selection Worksheet – Answer Key

Name: Instructor Copy  Date: ____________

# 📄 Part Overview

1. Part Name: 3D Part Design – Part 15

2. Material: Assumed Mild Steel or Aluminum (common choice for prototypes)

3. Part Type: ☑ Square (Prismatic) ☐ Round (Cylindrical)

4. Dimensions:
- Length: 140 mm
- Width: 60 mm
- Height: 60 mm

5. Tolerances:
- General Tolerance: ±0.1 mm (assumed)
- Critical Features: Hole locations, slot widths, stepped heights

# 🔍 Feature Identification

6. List and describe all features present in the part:

| Feature No. | Feature Description | Location on Part | Notes |
| --- | --- | --- | --- |
| 1 | Through holes Ø10 mm | Top face (3X) | Use spot drill and drill bit |
| 2 | Slot (24 mm wide, 30 mm deep) | Center top | Use end mill |
| 3 | Step profile (multiple heights) | Entire front and side | Mill in passes |
| 4 | Radius cut R5 | Upper right corner front face | Use corner rounding end mill or hand deburr |
| 5 | Vertical hole Ø10 mm | Front slot wall | Drill from side using vice fixture |

# 🛠 Tooling Selection

7. For each feature identified, determine the machining operation and select appropriate tooling:

| Feature No. | Machining Operation | Tool Type | Tool Size | Notes |
| --- | --- | --- | --- | --- |
| 1 | Drilling | Drill Bit | Ø10 mm | Spot drill first for accuracy |
| 2 | Pocketing | End Mill | Ø12 mm | Contour profile with step-downs |
| 3 | Facing & Slotting | Face Mill / End Mill | Ø50 mm / Ø10–12 mm | Multiple height passes |
| 4 | Corner Radius | Corner Rounding End Mill | R5 | Optional deburr if tooling unavailable |
| 5 | Side Drilling | Drill Bit | Ø10 mm | Align in vice from side |

# 🗂 Machining Plan

8. Outline the step-by-step machining process:

1. Face the top and sides to ensure square and flat stock.

2. Mill the stepped profiles on the front face.

3. Pocket the center slot (24 mm wide × 30 mm deep).

4. Drill the three Ø10 mm holes on top face.

5. Drill side Ø10 mm hole into center slot wall.

6. Mill R5 corner radius (optional or hand deburr).

7. Deburr edges and inspect critical dimensions.

# ✅ Final Checks

9. Inspection Plan:
- Measuring Instruments Required: Calipers, height gauge, depth micrometer
- Critical Dimensions to Verify: Slot width and depth, hole positions, overall height

10. Surface Finish Requirements:
- Specified Finish: Not defined (assumed 63 µin)
- Achieved Finish: Depends on tooling and material (face mill recommended)