# 🔍 Blueprint Analysis & Tooling Selection Worksheet – Answer Key (Part 19)

Name: Instructor Copy  Date: ____________

# 📄 Part Overview

1. Part Name: 3D Part Design – Part 19

2. Material: Assumed Mild Steel or Aluminum

3. Part Type: ☑ Round (Lathe) ☑ Square (Mill) – Hybrid part

4. Dimensions:
- Diameter: Ø52 mm base
- Height: 42 mm
- Mounting Base: 52 x 20 mm

5. Tolerances:
- General Tolerance: ±0.1 mm (assumed)
- Critical Features: Ø32 bore, bolt circle pattern, angular flats

# 🔍 Feature Identification

6. List and describe all features present in the part:

| Feature No. | Feature Description | Location on Part | Notes |
| --- | --- | --- | --- |
| 1 | Through bore Ø32 | Top to bottom | Lathe operation |
| 2 | Outer diameter Ø52 | Cylindrical body | Turned in lathe |
| 3 | Counterbores Ø20 x 15 | Top face, 4X | Drill + counterbore mill |
| 4 | Threaded holes M3 | On Ø25.5 PCD | Tapped manually or with tap guide |
| 5 | Angular flats 2X 30° | Sides of round boss | Milled with indexing head or rotary table |

# 🛠 Tooling Selection

7. For each feature identified, determine the machining operation and select appropriate tooling:

| Feature No. | Machining Operation | Tool Type | Tool Size | Notes |
| --- | --- | --- | --- | --- |
| 1 | Boring | Lathe Boring Bar | Ø32 mm | Use center drill, drill, and finish bore |
| 2 | Turning | Lathe OD Tool | Ø52 mm target | Use roughing and finishing tools |
| 3 | Counterboring | End Mill + Spot Drill | Ø20 mm | Mill to depth of 15 mm |
| 4 | Drilling + Tapping | Drill + Tap M3 | 2.5 mm drill | Use drill press or manual tapping |
| 5 | Milling flats | End Mill | Any suitable | Rotate part using rotary table or indexing fixture |

# 🗂 Machining Plan

8. Outline the step-by-step machining process:

1. Face and turn Ø52 diameter in the lathe.

2. Center drill, drill through, and bore Ø32 hole to depth.

3. Part off from lathe and clean faces.

4. Mount on mill, spot and drill counterbore holes (4X Ø20 mm × 15 mm deep).

5. Drill and tap M3 threads (4X) on Ø25.5 mm bolt circle.

6. Use rotary table to index and mill 30° flats on round boss.

7. Deburr and inspect.

# ✅ Final Checks

9. Inspection Plan:
- Measuring Instruments Required: Calipers, micrometer, protractor, thread gauge
- Critical Dimensions to Verify: Ø32 bore, bolt circle holes, counterbore depth

10. Surface Finish Requirements:
- Specified Finish: Not stated (assume 63 µin)
- Achieved Finish: Turned/milled as appropriate.