# 🧠 Machining Process Planning Worksheet – Answer Key

Instructor Copy – Suggested Answers and Guidance

# 🔧 PART TYPE

Suggested: Round = Lathe (e.g., shafts, bushings), Square = Mill (e.g., blocks, brackets)

# 📐 MATERIAL AND SIZE

1. Common materials: 6061-T6 Aluminum, Mild Steel, Delrin
2. Raw stock size example: 1” Ø bar for round parts, 2” x 4” plate for mill
3. Final size: Based on print and tolerance needs

# ⚙️ MACHINE SETUP PLANNING

4. Choose based on part geometry and tolerance
5. Lathe: Chuck, Collet; Mill: Vise, Fixture
6. Stick-out: 0.125"–0.250" max to prevent chatter

# 🛠 TOOL SELECTION

Example:

| Tool # | Tool Name | Use Case |
| --- | --- | --- |
| 1 | Face Mill | Create smooth flat surface |
| 2 | End Mill | Cut perimeter and slots |
| 3 | Drill Bit | Create holes |

# 🗂️ ORDER OF OPERATIONS

1. Face the part
2. Cut to length
3. Drill hole
4. Tap thread / Chamfer
5. Rough OD or Profile
6. Finish Cut
7. Deburr / Clean

# 🧠 STRATEGY QUESTIONS

9. Z0 at top face or face of part
10. Yes – if access needed on other side
11. Yes – tight tolerance surfaces
12. Calipers, micrometer, indicator, height gage

# ✅ FINAL CHECKLIST

| Step | Notes |
| --- | --- |
| Proper Workholding Secured | Check part secure & level |
| Tooling Installed & Touched Off | Touch-off with toolsetter or paper |
| Z0 and G54 Set | Match Z0 to setup |
| Program Simulated or Reviewed | Use backplot/single block |
| Coolant Adjusted | Aim nozzle at tool tip |
| Part Dry Run or Verified | Z lifted dry run for safety |
| Part Meets Blueprint | Use print and gauge to verify |